Alloying. What does alloying do?. Pure metals are rarely used in manufacturing because they are too soft. Usually, other elements are added to the molten metal so that the resulting solid is harder and has other desirable properties. An exception to this is is in the manufacture of car bodies which are made from new steels that are nearly pure iron. Adding larger ions In the alloy, some of the added ions may be larger than most of the ions making up the metal lattice. They disrupt the regular arrangement of ions and make it moredifficult for the layers to slide over each other. This makes the alloy harder and less malleable and ductile than the pure metal (in which the layers slip over each other more easily). Adding smaller atoms Smaller sized atoms can also have a significant effect on the alloy structure. In steel, for example, atoms of non-metals such as carbon and nitrogen can fit into holes between the iron atoms. This also distorts the metal lattice and makes it more difficult for the layers to move over each other..

| Picture 2.8 Many of these car parts use carbon steels... | Carbon steels. The percentage of carbon has a dramatic effect on the properties of the material and therefore on the uses for which it is suitable: Type of steel; Percentage of carbon; Properties; Uses. Low carbon (mild steel); 0.07 – 0.25; Easily cold worked; Car bodies. Medium carbon; 0.25 – 0.50; Wear resistant; Rails and rail products: couplings, crank shafts, axles, gears, forgings. High carbon (carbon tool steel); 0.85 – 1.2; Strong and wear resistant; Cutting tools Railway lines. Cast iron; 2.5 – 3.8; Easy to cast but brittle; Pistons and cylinders.. |
| --- | --- |
| Picture 2.9 Railway tracks are made from high carbon steel. Some heavy duty crossings are made from manganese steels... | Alloy Steels. The addition of small amounts of other metals to make alloy steels changes the properties of the material even further and means that a steel can be manufactured that has exactly the right properties for its purpose. Alloying element; Properties given to steel; Uses. Cobalt; High magnetic permeability; Magnets. Manganese; Strong and hard; Heavy duty railway crossings.. Molybdenum; Maintains high strength at high temperature; High speed drill tips. Nickel and chromium; Resists corrosion; Surgical instruments. Titanium; Increased hardness and tensile strength; High speed tool steels, permanent magnets. Tungsten; High melting temperature, tough; Cutting and drilling tools. Vanadium; Strong, hard; Tools.. |